Marine Expert Group meeting

7 November 2013

# Applying Art. 6 of Habitats Directive to Fisheries

- Implementing fisheries measures for marine Natura 2000 areas is a management priority for meeting the conservation objectives of the areas. In that context clarification is specifically needed on the application of Art. 6(2) and 6(3) to fishing activities. As a follow up to previous related work (Common methodology for assessing fisheries impact), and on the basis of discussion at the Marine Expert Group meeting of 6 November 2008, an overview has been carried out of how Member States seek to comply with those provisions. This overview will feed into the preparation of an interpretation note on the matter. A possible structure for such an interpretation note is provided in Annex to this document.

The participants are invited to consider the attached overview and provide feedback on proposed content and structure of the interpretation note.

# Application of Article 6.2 & 6.3 of the Habitats Directive

# to fishing activities

THE N2K GROUP

European Economic Interest Group

2013

Contents

BACKGROUND	2

SUMMARY AND CONCLUSIONS	2

THE NETHERLANDS	6

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	6

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	7

UK	7

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	8

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	10

IRELAND	10

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	11

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	12

FRANCE	13

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	14

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	15

GERMANY	16

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	16

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	18

CROATIA	19

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	19

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	20

SPAIN	21

Managing Fisheries under Article 6.2 – avoiding deterioration etc.	21

Managing Fisheries under Article 6.3 -  plans & projects requiring AA	23

RESEARCH PROGRAMMES	23

This paper has been prepared for the Commission to present an overview of the approaches being taken by several Member States to apply Article 6.2 & 6.3 of the Habitats Directive in relation to fisheries. It does not constitute an endorsement by the Commission of these approaches.

# BACKGROUND

Article 6.1 of the Habitats Directive requires Member States to establish conservation measures which correspond to the ecological requirements of the habitats and species for which Natura 2000 sites have been selected. These positive and proactive interventions may be within the framework of a management plan and can include statutory, administrative or contractual measures.  Although Article 6.1 sets out a key requirement of the Directive in relation to Natura 2000, it also the case that Articles 6.1- 6.4 are interlinked because, taken together, they indicate various tasks required to safeguard the nature conservation interests of Natura 2000 sites.

Articles 6.2 makes provision for the avoidance of habitat deterioration and significant species disturbance. It requires Member States to be proactive by taking preventative measures to avoid deterioration and disturbance that could be significant in relation to the objectives of the Directive.

Article 6.3 and 6.4 of the Habitats Directive lay down the procedure to be followed for any plans and projects likely to have a significant effect on a Natura 2000 site. Such plans and projects require an Appropriate Assessment, and may only proceed if they will not adversely affect the integrity of the site or, in exceptional circumstances, in spite of a negative assessment, and provided there are no alternative solutions, and the plan or project is considered to be of overriding public interest.

Where it is considered that the activity is not a plan or project in the sense of Article 6.3, Member States must nevertheless ensure the species and habitats for which a site has been designated do not deteriorate in accordance with Article 6.2. If the activities are not directly connected with or necessary to the management of the site (in line with Art 6.1), then an Appropriate Assessment may not be necessary either.

The Commission has provided general guidance on the provisions of Article 6 however some Member States are requesting specific assistance in determining the applicability of Article 6.2 & Article 6.3 to fishing activities. DG Environment, is preparing a clarification note on this point. As a contribution to this work, N2K have been asked to collate and present an overview of how several Member States are seeking to comply with Articles 6.2 & 6.3.

This paper is based on information provided by the Commission, some informal discussion with Member States where possible and internet searches. Further investigation may be required to get a more detail understanding of the how Member States are addressing this question both in terms of approach and practical application. At the same time, the findings to date have identified a number of issues that could usefully be covered in the clarification note.

# SUMMARY AND CONCLUSIONS

Several Member States have requested further interpretation of the Habitats Directive in relation to fisheries and also clarification of the process by which the management of fishing activities in Natura 2000 sites links to the CFP. The Commission has provided some guidance to help improve this understanding but is also preparing a further clarification note on the application of Article 6.2 and 6.3 to fisheries activities using existing jurisprudence and Case law. The material presented in this paper will help inform that clarification.

An overview of the current approach to application of Articles 6.2 & 6.3 in relation to fishing activities in Natura 2000 sites is provided for seven Member States; Croatia, France, Germany, The Netherlands, Ireland, Spain and  the United Kingdom. This is based on correspondence between Member States and the Commission (DG Environment), informal discussion with some Member States and internet searches. Although other Member States are also examining this issue there was insufficient documentation on their approaches to include them in this overview.

Member States are at different stages in their application of Articles 6.2 & 6.3 in relation to fishing activities. Several Member States have set out their processes, undertaken assessments, and introduced measures for fisheries management in Natura 2000 sites. However, it should be noted that some of these processes have evolved or are continuing to evolve, especially in light of national reviews or legal opinion. In other cases the implementation of Articles 6.2 and 6.3 in relation to fishing activities in marine Natura 2000 sites has still to commence.

A number of research programmes have provided a focus for gathering data and involving stakeholders in developing proposals for the management and regulation of fisheries activities in Natura 2000 sites. They have supported essential work to underpin the application of Article 6.2 & 6.3 in relation to fisheries in several countries (e.g. Germany, Netherlands, Spain). Nevertheless, there remains the difficulty of having sufficient and appropriate data to inform the setting of conservation objectives, and deciding on conservation measures which correspond to the requirements of the species. In the latter case collecting as well as accessing existing data on fishing activities within and around Natura 2000 sites is an issue for Member States.

The documentation reviewed for this paper suggests that at the present time no Member States have completed the entire process from assessment of potential impacts through to the full introduction of the necessary measures to manage fishing activities in all their marine Natura 2000 sites.  There also remains a need to confirm whether the processes and actions that have been introduced so far are fully compliant with the requirements of Articles 6.2 & 6.3.

The management of fishing activities in Natura 2000 sites is mostly being pursued under the scope of Article 6.2 i.e. the requirement to avoid deterioration and disturbance of the habitats and species for which the areas have been designated. This is generally taking the form of a “risk assessment” which sets out the likelihood of particular fishing methods having an impact on habitats/species of interest and the relevant conservation objectives.

Generic and site specific risk assessment matrices are being used to comply with Article 6.2 and the findings used to formulate management proposals. These might include restricting fishing activities to or away from particular areas or to set time periods. There may also be catch limits and required modifications if using particular types of fishing gear. The results of these risk assessments may be incorporated into site management plans or promoted by other means.

Where management plans have been prepared they typically provide a summary of the conservation measures which are being established to comply with Article 6.1 as well as showing the link to conservation objectives. There is also typically reference to the underlying data and evidence used to determine what measures are required. The proposed fisheries measures are being derived mainly on the basis of risk assessments and being implemented through actions such as zoning. To date, there has been limited consideration of cumulative and in-combination effects of fisheries measures with measures that are not concerned with commercial fisheries.

When Member States consider a fishing activity to be a plan or project in the sense of Article 6.3, the Appropriate Assessment procedure is undertaken. To date, these are mostly concerned with aquaculture (e.g. shellfish lays and oyster culture) although some mobile fisheries, usually those which require specific licenses have also been considered under Article 6.3.

Different approaches are being used as regards the application of Article 6.2 versus Article 6.3. In particular there continues to be a debate in some Member States about what type of fishing activity constitutes a plan or project and should therefore be subject to Article 6.3. There are some apparent discrepancies which merit clarification.

There also appear to be differences in interpretation of the links between Article 6.2 & 6.3. Some Member States (e.g. UK) have taken the view that a risk assessment under Art 6.2 does not necessarily remove the need for an appropriate assessment under Art 6.3. Other Member States do not appear to envisage any links (e.g. France according to ongoing communication with the Commission).

Some Member States have completed ‘strategic’ appropriate assessments in relation of  fishing activities and introduced regulations in response to these assessments where considered necessary, Examples referred to in this paper are from  the Netherlands (e.g. Seabed Protection Areas with restricted access for beam trawlers in Voordelta confirmed by Commission Decision 2008/914/EC); the UK. (e.g. closed areas for the hand worked cockle fishery in the Wash formalised through the Wash Fishery Order Regulations) and Ireland (e.g.  Castlemaine Harbour where a procedure is set out to determine the annual area to be fished for seed mussel, typically less than 50% of the historic extent of the main bed, but no additional regulation).

There remains the question of how far to go with ‘generic assessments’ and whether these could be considered sufficient to not require Appropriate Assessments under Article 6.3 or whether their purpose is rather to facilitate the AA process, for instances by helping to screen out projects early on so that full AAs are not necessary. There is relevant Case Law on this issue however the interrelationship between Article 6.2 and 6.3 could usefully be elaborated further, especially as regards the first initial screening test which determines whether there is a risk of significant effects (in which case an AA is required).

Whilst measures are being introduced to comply with Articles 6.2 & 6.3 there remains the need to examine whether, when taken together they are enough to comply with Article 6.1 in establishing the conservation measures or whether additional positive and proactive management measures are needed in relation to fisheries activities in Natura 2000 sites. Also, it is important to clarify what is meant by plans or projects ‘directly connected with or necessary for the management of the site’ as such plans and projects can also systematically be excluded from the need to carry out AAs under Article 6.3.

Potential issues for further elaboration in a clarification note.

- What are the requirements of Article 6 of the Habitats Directive?

- Overall

- Articles 6(2) & 6(3)

Commonalities

Key Differences

Relationship between

- How have key words in Article 6(2) been applied to fisheries?

- ECJ rulings relating to key words in Article 6(2)

- ‘appropriate steps’ in relation to fisheries

- ‘avoidance’ in relation to fisheries

- ‘deterioration and disturbance’ in relation to fisheries

- ‘significance of effects’ in relation to fisheries

- How have key words in Article 6(3) been applied to fisheries?

- ECJ rulings relating to key words in Article 6(3)

- ‘plan or project’ in relation to fisheries

- ‘not directly connected with but likely to have a significant effect thereon’ in relation to fisheries

- ‘appropriate assessment’ in relation to fisheries

- ‘adverse effects on site integrity’ in relation to fisheries

- Links between Article 6(1) conservation measures and fisheries management measures

- Plans and projects in the context of fisheries

-       What constitutes a fisheries plan or project

-       Types of activities which have been identified as fisheries plans or projects

-       The significance of fishing methods in determining plans & projects

- The linkage between Article 6(2) & 6(3) in relation to fisheries

- What are the opportunities for streamlining the application of Article 6(2) & 6(3)?

- Generic risk assessments

- Systematic screening

- Specific general conditions

- Introduction of required measures to comply with Articles 6(2) & 6(3) under the CFP

- The 11 point guidance

- Obligations under Articles 12 & 17 of the revised CFP [Commission & member states]

- Compliance with Article 6(2) & 6(3) for different types of fishing activity

- Appropriate Assessment for mobile gears (& unplanned activities?)

- What methods could be considered when undertaking appropriate assessment for wide ranging species?

- How might non-fisheries activities be considered alongside fisheries to determine in-combination effects?

# THE NETHERLANDS

In the Netherlands data gathered under the ‘Fisheries Measures in Protected Areas Project’ (FIMPAS) which ran from 2009-2011 has been key to informing and developing Dutch proposals for the management of fisheries in three offshore Natura 2000 sites. A separate initiative (the VIBEG agreement) has been undertaken for inshore areas and resulted in an agreement of principles, measures and commitments for fisheries measures in the first management plan period for two coastal zone Natura 2000 sites. Although not legally binding it contains a policy obligation for the parties concerned to adopt the agreed measures and to comply with them after their adoption.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

The FIMPAS project undertook an assessment of the extent to which fishing activities would threaten achieving the conservation objectives of Natura 2000 sites. The starting point was the recommendations for conservation objectives in the Dutch sector of the North Sea. Three workshops were held as part of the project. The first was concerned with establishing the data basis on which to carry out subsequent work. This involved documenting existing data, reviewing existing knowledge on the ecological impacts of fisheries prior to the workshop and incorporating additional data brought to the workshop. The second workshop undertook an assessment of the impact of fisheries on the designated sites with a particular focus on the overlap of time and space between fisheries and conservation objectives. The final workshop was used to generate management actions to meet the defined conservation objectives in line with agreed principles.

The FIMPAS project covered the Frisian Front (SPA) Cleaver Bank & Dogger Bank (SACs). Outcomes were an agreement on the appropriateness of a ban on gill net fisheries for Frisian front for 6 months of the year. Opinions differed for Cleaver Bank and in the end the Steering Group presented a zoning proposal covering beam trawling and bottom contact fisheries. Agreement on Dogger Bank management measures is being pursued through an international forum (Dogger Bank Steering Group) involving other Member States that also have designation parts of the Dogger Bank as a Natura 2000 site. Agreement between the parties was reached in May 2013, and subject to a legal process required by the Dutch Government, a request for regulation to the European Commission is pending.

Fisheries management measures to ensure compliance with the Habitats Directive have also been introduced for coastal Natura 2000 sites following consideration of potential impacts. These have been proposed under the 1998 Nature Conservation Act. In the Voordelta they comprise Seabed Protection Areas with restricted access for beam trawlers; creation of 5 rest zones for birds and seals and were part of compensation measures for harbour facilities in Voordelta. The measures were confirmed by Commission Decision 2008/914/EC.

For the North Sea Coastal Zone and the Vlakte van de Raan parties have agreed on zoning and regimes of measures including licence requirements for Dutch fishing vessels and entry notification requirement for foreign vessels. The measures were drawn up following the VIBEG agreement which set out a package of principles, measures and commitments to guide the management of fisheries in these two Natura 2000 sites. The measures are in place for the first management plan period for these sites (expected to be up to 2019). They are legally binding and supported by Commission Decision 2012/638/EU.

The assessments of the potential impact of fishing activities on Natura 2000 features through the FIMPAS project and VIBEG agreement were focused on the main fishing activities in the area independent of whether they were considered to be plans or projects.

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Under the Dutch Nature Conservation Act  (Natuurbeschermingswet 1998) any applications for licences of plans or projects which may have significant effects on the natural environment require an assessment in order to be compliant with Art 6.3 of the Habitats Directive. The assessment needs to be submitted by the applicant with the licence application and, following discussion with the relevant administrations and third parties, a decision will be taken with regard to the licence. Decisions can be challenged and final determination may be made by a tribunal.

A ‘prior authorisation procedure’ is being undertaken by the Dutch administration regarding the licensing of Dutch fishing vessels using a beam trawl with tickler chains or pulse trawl fishing in the offshore SACs and the Voordelta SPA. The Dutch Fisheries Board and others submitted a request for a licence for all vessels operating such gears in these sites with an appropriate assessment. The conclusion was that this type of fishing would have significant negative effects on the sand bank habitat and therefore measures were proposed to mitigate these significant effects e.g. zoning and from 2016 no fishing with tickler chains in the Natura 2000 sites.  The appropriate assessment was subject to scientific review which reached a similar conclusion about removing the risk of significant effects.  The outcome of this strategic level appropriate assessment is a licensing scheme for Dutch and Belgium shrimp trawling vessels and a reporting duty for all foreign vessels operating beam trawls with tickler chains in Dutch territorial waters. This system therefore combines consideration of the potential impacts of these fisheries on Natura 2000 features and a mechanism for management through licensing and reporting.

# UK

Fisheries management measures are dealt with by national governments within territorial waters and by the Department for Environment Food and Rural Affairs (Defra) and Marine Scotland, through the CFP, for waters out to 200nm.

Defra have been taking forward a revised approach to managing fishing activities in European Marine Sites in England and its offshore waters since August 2012. A ‘Fisheries in European Marine Sites Implementation Group’ has been set up to put this into practice, chaired by Defra and with representatives from the Marine Management Organisation (MMO), Inshore fisheries & conservation agencies (IFCAs), statutory nature conservation bodies, Seafish, environmental organisations and the fishing industry. The process for assessment of fisheries impacts is based on the use of best available evidence, with measures to be introduced on a ”risk-prioritised basis”.

This approach is being applied to sites within territorial waters around England and the measures will be applicable to both UK and non-UK fishing vessels.  For sites between 12-200nm fisheries management measures will be introduced through European fisheries legislation under the CFP.

A similar process is underway in Scotland based on advice from Scottish Natural Heritage to Marine Scotland. A Feature Activity Sensitivity Tool (FEAST) has been developed and is being used as the starting point for considering management options within a stakeholder consultation process. Potential management measures being considered through this by this process include those relating to fisheries.

A matrix approach is also being developed in Wales to map habitat sensitivity against types of fishing at different levels of activity (FishMap Môn). Although not specific to Natura 2000 sites and features this approach can be applied in such circumstances.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

For Natura 2000 sites in territorial and offshore waters around England the work overseen by the Fisheries in European Marine Sites Implementation group is intended to ensure that all existing and potential commercial fishing activities will be subject to an assessment of their impact on European Marine Sites (EMS). Where there is a significant risk conservation measures will be introduced by MMO/IFCAs.

Emergency byelaws will be implemented if the risk to the feature or sub-feature by a fishing gear type is highly likely and management needs to be implemented immediately. The MMO will also collate a list of sites that require legislative measures through the CFP The current timetable is to submit such proposals to the EC by the end of 2014.

A generic matrix has been developed to provide regulators with an indicator as to whether;

The activity requires priority management measures to be introduced to protect features without further site level assessment on the impacts of that activity on that feature

or

A further assessment may be necessary.

Fishing activities are classed as Red, Amber, Green or Blue according to the potential or actual impact of the gear type on the feature(s) for which a site has been designated and this determines what action will be taken (Figure 1).

Figure 1  Classification  scheme of the potential or actual impact of fishing gear of Natura 2000 features.

RED: Where it is clear that the conservation objectives for a feature (or sub-feature) will not be achieved because of its sensitivity to a type of fishing, - irrespective of feature condition, level of pressure, or background environmental conditions in all EMSs where that feature occurs - suitable management measures will be identified and introduced as a priority to protect those features from that fishing activity or activities.

AMBER: Where there is doubt as to whether conservation objectives for a feature (or sub-feature) will be achieved because of its sensitivity to a type of fishing, in all EMSs where that feature occurs, the effect of that activity or activities on such features will need to be assessed in detail at a site specific level. Appropriate management action should then be taken based on that assessment.

GREEN: Where it is clear that the achievement of the conservation objectives for a feature is highly unlikely be affected by a type of fishing activity or activities, in all EMSs where that feature occurs, further action is not likely to be required, unless there is the potential for in combination effects7.

BLUE: For gear types where there can be no feasible interaction8 between the gear types and habitat features, a fourth categorisation of blue is used, and no management action should be necessary.

Notes;  In theory, Green and Amber categories exist along a continuum of risk, for example it is theoretically possible for what is widely recognised to be a benign activity to cause a significant impact on a feature, if carried out at extremely high levels. As result, an assumption has been made that activities categorised as Green are assumed to have no impact on the feature or sub-feature at current maximum levels of commercial fishing effort, to be re-evaluated if conditions change.  In other words, where a type of fishing activity which on its own would not affect the achievement of conservation objectives for the feature, but which in combination with other activities might do so.  On this basis – where there is a feasible (even if unlikely) interaction, activities will be categorised as Red or Amber or Green.

Twenty five high risk European Marine Sites have been identified (19/6/13). The intention is for all high risk sites (Red) to have management measures in place (through regulation) by the end of 2013. The timetable for high priority ambers is the same as for red in that management measures will be identified by December 2013. For all medium and low risk activities (some Amber and all Green interactions on the matrix) the intention is for necessary management measures to be in place by 2016.

The work is focusing first on sites that contain features where evidence suggests there is significant risk that certain types of fishing activities could prevent a qualifying feature or sub-feature from achieving its conservation objectives.

In Scotland the Feature Activity Sensitivity Tool (FEAST) involves an assessment of the risk to features in nature conservation marine protected areas (including marine Natura 2000 sites and SAC features) and is the starting point for determining potential management requirements including those concerning fisheries. Using FEAST and following stakeholder consultation, Scottish Natural Heritage will provide advice to Marine Scotland on potential management measures or management questions requiring further discussion.

There is scope for features to be fast tracked based on the advice from SNH and for this to be followed up with detailed advice for particular locations.

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Fisheries already subject to a separate consenting process (e.g. Regulating Orders) are being dealt with under Art 6.3. This includes the majority of shellfish lays, trestles and shellfish culture and a number of appropriate assessments have been carried out to that effect.

There are examples where a strategic approach has been used to assist with Article 6.3 assessments by agreeing broad guiding principles and associated management measures in advance. In the Wash Natura 2000 site, this has enabled agreements on the annual fishery for cockles and mussels to proceed more rapidly whilst still fulfilling appropriate assessment as required under Article 6.3.  Generic fisheries management measures consistent with the conservation objectives for the Natura 2000 site, such as TACs for each annual fishery based on a stock survey, and the method by which cockles and mussels can be fished were agreed in 2008. With that agreement the fisheries authority and conservation agencies responses to the appropriate assessments and agreement of the annual plans for cockle and mussel fisheries have become more streamlined and routine whilst continuing to comply with the Art 6.3 requirements.

Fishing activity likely to be prohibited or restricted in view of the generic risk assessment (described above for England) could be allowed through a permitting mechanism at the site level, but will be subject to Art 6(3)-(4) processes (e.g. a screening stage will still be required to confirm no risk of significant effects). Article 6.3 procedures are therefore being recommended for activities/feature interactions which are graded as Amber or Green in the above process.

# IRELAND

The roadmap to implementation of the Habitats and Birds Directives in Ireland  indicated that the assessment of the impact of fishing activities on designated features would be undertaken under Article 6.3. More recently (2013), and in light of lessons learnt from Appropriate Assessments already undertaken, Ireland propose to consider inshore fishing activities under Article 6.2 in the first instance.

Risk assessments are due to be carried out for all Natura 2000 sites with published conservation objectives in order to determine what management measures are necessary. These assessments will also be used to determine whether any fishing activities can be screened out because they pose no risk to Natura 2000 sites. The assessments will be undertaken using a qualitative framework, and semi-quantitative data, as this is often what is available in relation to fisheries. They will be specific to the conservation objectives for each site and to the conditions (e.g. features subject to conservation objectives, fishing effort and distribution) in each site.

Periodic review of fishing operations and fishery plans is proposed regardless of whether fisheries are being considered under Article 6.2 or Article 6.3.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

Ireland is developing a framework for the assessment of risk posed by fishing activities in its Natura 2000 sites in order to comply with Article 6.2. This is intended to provide a flexible, iterative and adaptive approach to identifying the steps that should be taken in relation to inshore fishing activities and the conservation objectives of individual SACs.

The starting point is bringing together data on the footprint of the fishing activity, its duration or frequency, and its intensity. This will be needed for each fishing metier and in relation to each habitat in order to assess the risk that each activity poses to each habitat (a combination of the consequence and likelihood). Figure 2 illustrates how this approach might be summarised in a matrix however it should be noted that when putting this into practice the assessment needs to be tailored to the conservation objectives and characteristics of the fishing activity within the site. For example in appropriate assessments of fisheries and aquaculture in Dundalk Bay a threshold of 15% of habitat is used as a criterion for significance.

Figure 2. Illustration of a structure which can be used to give an overview of a risk assessment of a specific fishing activity in relation to a specific habitat and its associated conservation objectives. NB. Criteria thresholds and the likelihood of risk will need to be determined on a site specific basis. Table 3 from Marine Institute (in press)

A similar approach is proposed when considering species listed in Directive although the risk assessment framework also needs to cover ex-situ aspects such as the significance of the impact of a particular fishing activity on populations outside the designated site.

The risk assessment process will start with bringing together and interpreting include expert judgement risk assessment workshops and conclude with risk profiles for each site. A hypothetical example is included as Figure 3.

Figure 3. Hypothetical profile of risk to designated features due to fishing activities in a Natura 2000 site. The number of incidences is from a conflict matrix which identifies all possible interactions (habitat * fisheries and species * fisheries). The risk profile is a point in time estimate and may change due to reduction or escalation of activity. Table 5 from Marine Institute (in press)

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

The roadmap to implementation set out by Ireland indicated that assessment of the impact of fishing activities on designated features in Natura 2000 would be undertaken under Article 6.3.

A phased process of appropriate assessment and risk assessment was initiated under the guidance of the Department of Agriculture, Food and the Marine and four sites (Castlemaine Harbour, Dundalk Bay, Roaringwater Bay and Donegal Bay (Murvagh)) have been subject to such assessments to date. The process started with the gathering of scientific data on the habitats and species, fishing activities and setting conservation objectives. This was followed by the preparation of a Fisheries Natura Plans (FNP) which were submitted to the Minister and to the Marine Institute for assessment.  Appropriate Assessment (AA) of the effect on the site of the proposed fishing activity were also undertaken and conclusions drawn on the likelihood of significant effects on the habitats and protected species arising from the proposed fishing activity. By considering the entire Bay, these assessments have also been useful way of considering in combination as well as individual effects.

Determination of significant effect is based on three parameters; disturbance, the persistence of such disturbance and the area of habitat affected. In the case of aquaculture in Dundalk Bay, a cut-off figure of 15% has been derived having considered the Article 17 reporting framework where disturbance of greater than 25% of the area of an Annex 1 habitat represents unfavourable conservation status.  If all three elements are true then the proposed activity has to be mitigated or it cannot proceed.

Appropriate Assessments as required by Article 6.3 have also been undertaken for several mobile fisheries in the Dundalk Bay SAC e.g. hydraulic dredging for cockles, proposed dredging for mussels and beam trawling for brown shrimp in the fine sand community complex.

After public consultation on the FNP and AAs and review by an adhoc Technical Advisory Committee the documents are approved by the Minister. If the Minister considers the management measures must be given force of law a legal instrument (“Natura Declaration) makes such measures mandatory. This can restrict access to particular fisheries to permit holders and which may themselves have conditions.

Fishing activities are reviewed periodically to determine if any changes to the FNP or management measures needed. Where AAs are needed but have yet to be carried out, Inland Fisheries Ireland has stated that (at least in the case of the wild oyster fishery which requires a licence) in that no intensification of the fishery must take place.

# FRANCE

The approach to application of Articles 6.2 & 6.3 of the Habitats Directive in France has been transposed into national legislation in a Code de l’environnement (Environment Code).  Following a judgement in the European Court of Justice in 2010 (Case C-241/08) the law was amended to make it clear that although activities (such as fishing) may be covered in the scheme of management for a site which has been tested against the conservation objectives, this does not systematically exempt them from being subject to appropriate assessment under Article 6.3.

Article 6.1 requires the establishment of necessary conservation measures and in France the relevant information is brought together in a scheme of management for each Natura 2000 site, known as the ‘document d’objectifs’ - the DOCOB. This provides the framework for identifying and implementing conservation measures and because it is drafted by a steering committee of stakeholders it is a fully participative process. The conservation objectives and suggested management measures for the site are set out in the DOCOB as well as an inventory of the habitats and species which justified the designation of Natura 2000 site, and an inventory of human activities taking place on the site including identification of their effects on the site features. Where effects are negative, changes need to be proposed to ensure the conservation and restoration of the affected habitats and/or species. Proposals for site management are based on a risk analysis and subsequent prioritising of the risk.

The resulting management measures may be introduced using various legislative and financial instruments. These may use national regulation or individual Natura 2000 contracts with land owners and stakeholders although France has also raised the question of whether measures under the CFP are considered adequate to fulfil certain requirements of the Habitats Directive.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

France started to develop technical tools to support the inclusion of commercial fisheries in the DOCOB of Natura 2000 sites in 2009. The work, undertaken by a partnership of CNPMEM, IFREMER, the French MPA Agency, and the Environment Ministry, is intended to comply with Article 6.2 by virtue of taking appropriate steps to avoid deterioration of habitats and disturbance of species. There are three main elements;

- A technical reference on the activities of professional marine fisheries

- Factsheets for fishing data produced by the Marine Agency on the basis of surveys by Ifremer

- A scientific method of “risk analysis” as defined by the National Museum of Natural History (MNHN) , for the analysis of interactions between fisheries and issues/sites

This approach was seen as a way of determining the effects of commercial fishing activities in relation to the conservation objectives of Natura 2000 at each site as well as promoting consistency and prioritising proposals for the management of Natura 2000 sites.

The risk assessment for fisheries in marine Natura 2000 sites overlays three types of information: the occurrence of habitats, fishing (probability of occurrence and intensity), and information on interactions (habitat sensitivity and subsequent effects) (Figure 4). This will be used to identify and quantify the risks of different fishing activities on each habitat according to two parameters; the potential impact (high, moderate, low or zero) and the sensitivity of the habitat from a pressure (high, moderate, low or unknown). The analysis will be used to identify whether additional information is required to reach a conclusion or whether management measures are required. This is also seen as a way to inform and involve fishermen as they are contributing to the process.

There is a further step involving administrations and fisheries committees to ensure consistency in measures across sites and a proportionate approach. The results of the risk analysis will be incorporated into a draft DOCOB which needs to be finally approved by the Maritime Prefect and/or the Prefect of a Department.  Work is underway to prepare the fishing ‘factsheets’ and it is envisaged that the risk analyses for sites where the Agency for Marine Protected Areas is the operated will be completed by the beginning of 2014 and for all other sites during 2014.

Figure 4.  Schematic summary of the process for assessing the risks of degradation of habitats from fishing activities within Natura 2000 sites. (MNHN et al., 2012)

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Art L.414-1(V) of the Environmental Code states that “fishing, aquaculture, hunting and other hunting-related activities practiced under the conditions and in the areas authorised by the laws and regulations in force shall not constitute activities causing disturbance or having such an effect”.  The implications would be that there was no need for appropriate assessment of such activities (e.g. fisheries) under Article 6.3 if they are included in the DOCOB (a contract). This position was challenged by the Commission and the challenged was upheld (Case C-241/08 Commission v France). The judgement makes it clear that the fact that a contract for a site, such as the DOCOB, complies with the conservation objectives is not sufficient to enable all plans or projects provided for in those contracts to proceed automatically because it cannot guarantee that the activities do not adversely affect the integrity of the protected site. The ruling states that there is still a need for appropriate assessment in relation to the conservation objectives for the site.

To remedy this situation France has investigated a number of options and is proposing the following approach;

- Risk assessments of all professional fisheries in each Natura 2000 site under the DOCOB

- Where a significant risk of impact is identified, application of regulatory measures

- The application of such measures would exempt individual fishermen from the obligation of carrying out individual assessments

The advantages of this approach would be that regulatory measures could be tailored to the activities and conditions in each Natura 2000 site as well as being proposed as part of the DOCOB and therefore allowing consultation with local stakeholders. The French government have put forward some wording to enshrine this approach in the proposed “biodiversity” bill.

# GERMANY

In Germany, nature conservation, including the management of Natura 2000 sites is subject to federal jurisdiction in waters beyond 12nm, and to the respective nature conservation laws of the federal states Lower Saxony, Schleswig-Holstein and Mecklenburg-Vorpommern in territorial waters. In the Exclusive Economic Zone, fisheries management is in the joint (but still to be clarified) competence of the federal ministries of Environment, Nature Conservation and Nuclear Safety (BMU) and Food, Agriculture and Consumer Protection (BMELV).

Generally, the federal government of Germany and the respective coastal federal states consider fishing including shellfish fishing and cultivation under Article 6.2. According to German nature conservation legislation (BNatschG, §57.2), any measures taken to limit fishing activities (including those in Natura 2000 sites) have to conform with the the ”Seefischereigesetz“ the respective ordinance ”Seefischereiverordnung“, and in accordance with European fisheries regulations. Fishing activities carried out according to ”gute fachliche Praxis“ [good practice] (BNatschG 2009, § 5.4) are generally assumed not to contradict the aims of nature conservation (BNatschG, § 14.2).

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

EEZ

The process of developing fishery management measures in Natura 2000 sites in the German EEZ started in 2005. The project Environmentally Sound Fisheries Management in Protected Areas (EMPAS, 2006-2008), compiled fisheries data, provided a conflict analysis and produced site-specific management recommendations based on generic risk assessment. Site-specific conservation objectives were developed in parallel.

In 2009, a steering committee was set up by the competent German ministries, BMU and BMELV, together with their respective agencies the Federal Agency for Nature Conservation (BfN) and the Thünen Institute (TI) and the relevant ministries of the coastal Länder (Mecklenburg-Western Pomerania, Lower Saxony and Schleswig-Holstein) to ensure that the requirements of the Habitats Directive and the Birds Directive are implemented in the Natura 2000-sites in German EEZ within the prescribed period. A scientific working group (WG North Sea and Baltic Sea, consisting of representatives of the vTI and the BfN), was commissioned in May 2010. The group worked on points 1 through 9 and 11 of the European Commission's guidelines for the implementation of fisheries management measures in Natura 2000 sites. Site-specific recommendations for fisheries management measures, consistent with the conservation objectives and considering the generic risk of deterioration as well as minimising the socio-economic impact, were presented in 2011.

At the end 2011, the majority of fisheries ministers of the federal states requested a revision of the measures proposed for the EEZ and decided that measures are required only in case of proven significant negative impact of fishing activities on the relevant species and/or habitats subject to the Habitats and Birds Directive in the respective Natura 2000 sites. Any measures to be taken shall distinguish fishing métiers and seasonal intensity. As a rule, all those regular fishing activities which have existed at the time of site nomination have the right of continuance, unless measures lead to significant adverse impacts. National and international consultations are now complete and revised proposals due to be forwarded to the Commission. Changes to the CFP due to come into force in January 2014 concerning details of the required process (articles 12 & 17) may result in further delay.

In the case of the Dogger Bank SCI, Germany participated in the FIMPAS project initiated by the Dutch government and subsequently the intergovernmental Dogger Bank Steering Group in order to achieve an agreed fisheries management scheme for the whole of the bank [see section on Netherlands for further details].

Status in the Federal States

A variety of measures are in place to manage fisheries in coastal waters up to 12 nm from shore, including in marine Natura 2000 sites in the federal states. They have been established under state ordinances and procedures and include some site specific regulation, albeit concerned with the management of fisheries rather than nature conservation. Whilst they have not been developed in response to the management of Natura 2000 habitats and species or requirements of Article 6.2, these measures are what is currently available to fulfil the Article 6.2 obligations.

Schleswig-Holstein

In 2013, as part of the regular revision cycle of the Küstenfischereiverordnung (KüVO), Schleswig-Holstein aims to introduce area-based closures for the local set-net fishery in order to reduce the bycatch of harbour porpoises and diving seabirds. The areas in question will partly coincide with Natura 2000 sites but have not been developed with the management of particular Natura 2000 sites in mind. The revised ordinance is currently subject to national and international consultation and follows extensive stakeholder consultation.

In the Wadden Sea National Park of Schleswig-Holstein there are a variety measures concerned with the management of commercial fisheries including prohibitions of some fisheries eg. Spisula and oysters, and area based controls on others e.g. Mussel fisheries. The Wadden Sea Board has adopted set of principles for sustainable fisheries that will be on the agenda of the 12th Wadden Sea Conference in 2014.

Niedersachsen (Lower Saxony)

All marine Natural 2000 sites of Niedersachsen are within the boundaries of the Wadden Sea National Park. There is a zoning scheme and licensing of some fisheries in some areas but no specific assessment of fisheries in relation to Article 6(2).

Hamburg

All commercial fishing, including for brown shrimp and mussels is prohibited in the Hamburg section of the Wadden Sea National Park according to the National Park legislation (1990).

Mecklenburg-Vorpommern

Fisheries are generally regulated through the Küstenfischereiverordnung, (KüFVO M-V, 2009), which is currently in the process of revision. It is generally assumed that all those fishing activities in line with „good practice“ (acc. BNatschG 2009, § 5.4) which took place at the time of nomination can continue. According to the nature conservation law of Mecklenburg-Vorpommern (§ 12, Absatz 2 NatSchAG M-V, 2010) fishing activities may be exempt from restrictions in the ordinances which regulate activities in the protected areas.

Mecklenburg-Vorpommern implemented the first management plan for a coastal Natura 2000 site with marine components in 2009 (Wismar Bay). Further plans for other areas are being elaborated and negotiated according to a priority list. These management plans cover all activities including fisheries (e.g. Wismar Bucht, Greifswalder Bodden) however professional fishing is generally not considered to be inconsistent with the conservation goals.

In the state of Mecklenburg-Vorpommern measures are being initiated if the conservation status of Natura 2000 habitats has deteriorated with respect to the reference status at the time of nomination (2004-2009) and is considered unfavourable (C) or significant reductions in habitat area took place according to a national scheme.  As the first of these evaluation schemes was only published in 2011, no valid deteriorations are assumed and thus no recovery measures taken, only measures to improve the conservation status to favourable condition in the long term.

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Limited information was found on the application of Article 6.3 in relation to fisheries in Germany with the exception of a mussel cultivation in Flensburger Förde. One location where there may be further developments on this issue is the Wadden Sea. The Wadden Sea board are currently considering proposals for a consistent approach to sustainable fisheries in the World Heritage site which includes several Natura 2000 sites and extends across Denmark, Germany and the Netherlands.  There is agreement by the members that the use of regular appropriate assessments (or equivalent impact assessments) should be adopted by all Wadden Sea regions and applied to all fisheries. These are to be based on nature conservation objectives and be part of the drive to achieve sustainable fisheries. The nature of such action is still to be agreed with the issue on the agenda for the 12th Wadden Sea Conference in 2014. Annual appropriate assessments already take place in the Netherlands and Environmental Impact Assessments in Danish part of the Wadden Sea.

# CROATIA

Croatia only joined the EU in 2013 and has only very recently proposed pSCIs and SPAs. Existing experience of managing disturbance and potentially damaging activities in protected areas has been adapted to ensure compliance with Article 6.2 & 6.3 when the Natura 2000 sites are formally designated. The mechanism of Nature Impact Assessment (NIA) for the National Ecological Network which was agreed in 2007 has been harmonised with the Birds and Habitats Directive setting out the process for Appropriate Assessment and giving it a statutory basis. Experience of management and the measures taken to date within Marine Protected Areas (MPAs) is also relevant, particularly when considering the implementation of Article 6.2 in relation to marine Natura 2000 sites.

The scientific foundation on which to base the implementation of Article 6 of the Habitats Directive in marine Natura 2000 sites, is being built up. Work is underway to complete the mapping of Natura habitat types as a basis for defining precise conservation measures, including any regulation of fishing activities as appropriate. Using data collected through the EU supported project IPA 2007 and other sources, guidelines for the assessment of conservation status of marine habitat types have also been prepared. There is a detailed programme of work describing steps needed for implementation of the marine part of Natura 2000.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

Whilst marine Natura 2000 sites in Croatia have still to be designated and have no proscribed management measures as yet, the provisions under the Marine Fisheries Act and its subordinate legislation can be used to protect marine biological resources in accordance with Article 6.2. This legislation states that fishing activities in MPAs are subject to nature protection statutes. Management is through internal orders enacted separately for each site. Commercial fishing is forbidden in marine national parks and only small-scale traditional fishing is permitted for personal use of local people. Additionally, strictly regulated recreational fishing (angling) can be allowed in defined areas. In nature parks fishing is allowed but under regulation by internal order.

A number of MPAs have agreed management plans and these cover the issues of fishing and conservation of Natural 2000 target features. One example is the management plan of the Lastovo Archipelgo Nature Park which was adopted in 2013. The plan defines how fishing is regulated, based on scientific assessment of fishing resources from 2010. There are concession agreements with the Management Authority for traditional and recreational fishing, and zoning combined with permits is used to regulate effort and reduced impacts.

Marine fisheries legislation prescribes detailed conservation measures for biological resources, including: spatial and temporal regulation of fishing; defined technical features of the fishing gear; minimum landing size of commercially important species; limiting fishing effort; closed season; quota system; protection of special habitats for fish and other marine organisms.

With accession to the EU, some additional measures came into force. This includes measures to comply with Council Regulation (EC) No 1967/2006 concerning management measures for the sustainable exploitation of fishery resources in the Mediterranean Sea (so called ‘Mediterranean Regulation’) – e.g. harmonizing minimum sizes of marine organisms, preparing management plans for certain fisheries, prohibition of certain fishing gears in areas of seagrass beds, coralligenous habitats and maerl beds.

According to the Institut za oceanografiju i ribarstvo (IOR)  it is urgent to start with more restrictive measures regarding regulation of fishing efforts and conservation of biological resources, especially in the most important areas (open Middle Adriatic area and beyond territorial waters).

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Assessment of plans is at an early stage in Croatia and it is implemented through a strategic environment assessment procedure according to the National Park Act (NPA) and Environment Protection Act (EPA). The EPA states that all strategies, plans and programs adopted on national, regional and partly on local level (large cities) related to fisheries (among other sectors) are subject to the Strategic Environmental Assessment, which includes Appropriate Assessments, if these might affect the ecological network. A three stage process for AAs is set out in the new NPA (80/13); pre-assessment, the main assessment, and establishment of overriding public interest and compensatory measures. The AA is focused on the features for which the site has been designated and overall site integrity. SEA and EIA have a much broader scope.

Currently, there are SEA procedures for physical plans (relevant to defining locations for mariculture) as well as for strategic fisheries documents such as the National Strategic Plan for Fisheries and Operational Program for Fishery. In future all management plans based on the MFA will also be the subject of SEA/AA procedure.

There is a clear case for Appropriate Assessments under Article 6(3) for aquaculture sites and in Croatia this will be implemented through the Environmental Impact Assessment process which is obligatory according to the NPA and Environmental Protection Act. Proposals to enlarge fish farm capacity in the bay of Veli Bok on the island of Cres provide an example of an AA. The outcome was permission for the project to proceed, subject to mitigation and monitoring. As part of the decision making process, the Nature Protection Directorate determined that the project would have no significant influence on conservation objectives and integrity of the ecological network of sites.

# SPAIN

In Spain the LIFE+ INDEMARES project has been key to identifying potential fishing conservation measures in a number of proposed marine Natura 2000 sites and is therefore relevant to compliance with Articles 6(2) and 6(3).

The project, which is ongoing, has developed a methodology to assess the fishing footprint and establish appropriate conservation measures for 10 sites across the Atlantic, Mediterranean and Macaronesic regions (Cañón de Avilés, Banco de Galicia, Chimeneas de Cádiz, Seco de los Olivos, lsla de Alborán y conos volcánicos, Delta del Ebro-Columbretes, Cañón de Creus, Canal de Menorca, Banco de la Concepción and Sur de Fuerteventura).

The types of fishing activities, their spatial extent, overlap with marine habitats, and the sensitivity and vulnerability of those habitats to certain activities has been described for each of the ten locations. Sources of information included VMS data, fleet census, logbooks, fisheries surveys and on-board observers. Taken together this information was used to describe a ‘fishing footprint’. A systematic process, including technical workshops and conservation activities with fishermen was used to define management needs and conservation measures for each area. Monitoring schemes, predictive models and simulations to assess the spatial effects for the fleets concerned have also been developed through this project. The work is due to be completed by the end of 2013 when the findings will be presented to the Commission.

## Managing Fisheries under Article 6.2 – avoiding deterioration etc.

“El Cachucho" was declared as a Marine Protected Area (MPA) and Special Area of Conservation (SAC) by the Royal Decree 1629/2011. This includes references to the article 149.1.23 of the Spanish Constitution related to conservation in the marine environment. The fisheries management plan which is also part of the Decree (see below) is not covered by this provision but is drafted in accordance with Article 149.1.19 which gives the government exclusive competence on offshore fisheries. There is another link with legislation concerned with offshore fisheries (Ley 3/2001, 26 de marzo, de Pesca Marítima del Estado) the result of which empowers the Minister of Environment and Rural & Marine Areas to amend the fisheries management plan but under the framework of both this legislation and the CFP. These links embed fisheries management into the statute which establishes the MPA.

The Royal Decree also approves the management plan for the area which includes the conservation measures according to Article 6.1 of the Habitats Directive for all relevant activities, including fisheries (Annex II of the Decree).

The process for developing and adopting fisheries management measures for this SAC is summarised below as an example of how Spain is taking forward the application of Article 6(2) in relation to fisheries in Natura 2000 sites. Further examples will come on the conclusion of the INDEMARES project which aims to complete the identification of the marine Natura 2000 network in Spain as well as providing management guidelines (including those for fisheries) for each of ten proposed Natura 2000 sites.

Fisheries management in El Cachucho SAC

Conservation measures for the regulation of fishing activities in the El Cachucho SAC are set out in a Fisheries Management Plan. This is a component of the overall site management plan which is attached to the Decree that designated the SAC. The Fisheries Management Plan sets a general prohibition of the use of bottom trawling gears and regulates the provisions of special fishing permits for a number of vessels which targeted greater forkbeards (Phycis blennoides) using bottom longlines between 2006-2008.

The proposed conservation measures were defined by considering environmental values in combination with the sensitivity and vulnerability of the area to different fishing activities (See Figure 5). Firstly the spatial distribution of vulnerable habitats was defined. This was followed by detailed mapping which was mainly focused on ‘reef’ habitats (1170) and identification of fishing gears operating in areas covered by this habitat.

Figure 5. Flow diagram illustrating the various steps towards drawing up conservation measures in relation to fisheries and reef habitat for the El Cachucha SAC

Key data were collected during three oceanographic surveys carried out by the Spanish Institute of Oceanography (IEO) between 2003-2005 as part of the ECOMARG project (continental margin ecosystem and impact of fisheries). This was brought together with the results of previous research surveys and a broad range of scientific literature of the IEO, including the results of studies on fleet presence and distribution based on vessel monitoring system (VMS) data.  The findings were presented to the Spanish Ministry of Agriculture, Food and Environment (MAGRAMA) in May 2007 and followed by a stakeholder consultation process overseen by an interministerial commission.

Two working groups were set up: a scientific and technical working group and a legal administrative working group, composed of specialists representing environmental (national and regional), fisheries (national and regional), coastal management, protection of natural areas, maritime and foreign affairs administrations, fishermen’s associations, scientific institutions and marine environment conservation NGOs. These working groups held three meetings each from 2006 to 2008.

As well as examining the data, these meetings were an opportunity to discuss the wider implications of management on fisheries such as acceptance by the fishing industry of the designation of the area as MPA considering the presence of fishing fleet in the area, and the presence of breeding adults of some commercially valuable species such as blue whiting (Micromesistius poutassou) and therefore the potential to have a positive impact on the overall protection of fisheries in the Bay of Biscay and the Cantabrian Sea

Following the establishment of the fisheries conservation measures, a management agreement for the monitoring and assessment of the measures between the MAGRAMA and the IEO was signed. Monitoring of fishing activity has not commenced as none of the eligible vessels have applied for special permission to fish in the area. If this situation should change the agreement envisages various monitoring methods such as, observers on fishing vessels to assess the performance of the greater forkbeard for the catch of species of commercial interest, and the impacts on species of non-commercial interest; the use of VMS data of the area before and after its appointment as AMP to calculate the effort being undertaken in the area; experimental sampling with gears for monkfish (Lophius piscatorius) to assess the impact on communities. ROV surveys are planned to improve the resolution of spatial distribution of the Natura 2000 habitat 1170 “Reefs”.

## Managing Fisheries under Article 6.3 -  plans & projects requiring AA

Regarding Article 6.3 of Habitats Directive, Article 4 of Royal Decree 1629/2011 sets that the assessment of any plans, programmes and projects that might have a significant effect on the SAC shall comply with the provisions in Article 45 of Act 42/2007  concerning conservation measures for Natura 2000 Network. This article 45 includes all the provisions of article 6.3 and 6.4 of the Habitats Directive.

# RESEARCH PROGRAMMES

Several programmes of work have been particularly useful in helping to develop fisheries management measures in marine Natura 2000 sites. The main elements have been gathering data on fisheries, and assessment of fishing impacts on Natura 2000 features, both of which are necessary parts of complying with Articles 6.2 & 6.3 of the Habitats Directive.

- EMPAS (Environmentally Sound Fisheries Management in Protected Areas) 2006-2008.The German Federal Agency for Nature Conservation (BfN) in collaboration with ICES. The aim was to develop fisheries management plans for Natura 2000 sites within the German EEZ. Outputs include proposed fisheries management measures.  These measures are being agreed nationally and discussed internationally. Proposals will be submitted to the Commission to introduce regulations that will apply to both national and foreign fishermen.

- FIMPAS (Fisheries Measures in Protected Areas) 2009- 2011.   To develop fisheries measures for Natura 2000 sites in the Dutch North Sea. Proposals to be submitted to the European Commission to be covered by regulations that will apply to both national and foreign fishermen

- BALTFIMPA (Managing Fisheries in Baltic Marine Protected Areas) 2012.  (HELCOM in collaboration with ICES including application for Life-BALTFIMPA to run 2014-16 if successful).  Includes detailed assessment of fishing activities and their impacts within selected MPAs in the Baltic and drafting of fisheries management plans with reference to EU Guidance documents.

INDEMARES  (Inventory and designation of the Natura 2000 Network in marine areas of the Spain). Due for completion at the end of 2013.  Includes identification of conservation and monitoring actions for marine Natura 2000 and research studies in 10 proposed marine Natura 2000 sites.

ANNEX

Legal Interpretation Note

On the application of Article 6(2) and 6(3) of the Habitats Directive (92/43/EEC) in Natura 2000 marine sites with regards to fishing activities.

The purpose of this note is to help Member States gain a better understanding of the application of Article 6(2) & 6(3) as regards fishing activities. It complements the EC Interpretation Guide for Article 6 and should be read in conjunction with that document.

Proposed contents

- What are the requirements of Article 6 of the Habitats Directive?

- Overall

- Articles 6(2) & 6(3)

Commonalities

Key Differences

Relationship between

- How have key words in Article 6(2) been applied to fisheries?

- ECJ rulings relating to key words in Article 6(2)

- ‘appropriate steps’ in relation to fisheries

- ‘avoidance’ in relation to fisheries

- ‘deterioration and disturbance’ in relation to fisheries

- ‘significance of effects’ in relation to fisheries

- How have key words in Article 6(3) been applied to fisheries?

- ECJ rulings relating to key words in Article 6(3)

- ‘plan or project’ in relation to fisheries

- ‘not directly connected with but likely to have a significant effect thereon’ in relation to fisheries

- ‘appropriate assessment’ in relation to fisheries

- ‘adverse effects on site integrity’ in relation to fisheries

- Links between Article 6(1) conservation measures and fisheries management measures

- Plans and projects in the context of fisheries

-       What constitutes a fisheries plan or project

-       Types of activities which have been identified as fisheries plans or projects

-       The significance of fishing methods in determining plans & projects

- The linkage between Article 6(2) & 6(3) in relation to fisheries

- What are the opportunities for streamlining the application of Article 6(2) & 6(3)?

- Generic risk assessments

- Systematic screening

- Specific general conditions

- Introduction of required measures to comply with Articles 6(2) & 6(3) under the CFP

- The 11 point guidance

- Obligations under Articles 12 & 17 of the revised CFP [Commission & member states]

- Compliance with Article 6(2) & 6(3) for different types of fishing activity

- Appropriate Assessment for mobile gears (& unplanned activities?)

- What methods could be considered when undertaking appropriate assessment for wide ranging species?

- How might non-fisheries activities be considered alongside fisheries to determine in-combination effects?

E.g. Netherlands, France, Ireland

E.g. EC (2000) Managing Natura 2000 Sites. The provisions of Article 6 of the ‘Habitats’ Directive 92/43/EEC; EC (2012) Fisheries Measures for Marine Natura 2000 Sites. A consistent approach to requests for fisheries management measures under the Common Fisheries Policy; EC (2012) Guidance on Aquaculture and Natura 2000. Sustainable aquaculture activities in the context of the Natura 2000 Network; EC (2013) Establishing conservation measures for Natura 2000 Sites. Note to the Habitats Committee. Common Methodology for assessing the impact of fisheries in marine Natura 2000 sites.

E.g. Case C-127/02 Cockle fishing in the Wadenzee; C-241/08 Commission v.France;  C-418/04  Commission v Ireland; Case C-98/03 Commission v Germany  Case C-241/08 Case C-6/04, Commission v UK, C-392/96 Commission v Ireland Case C-304/05 Commission v Italian Republic,Case C-404/09 Commission v Kingdom of Spain, Case C-226/08 Stadt Papenburg v Bundesrepublik Deutschland,

E.g. France, Ireland, United Kingdom

E.g. some of the Federal States in Germany, Croatia which has only recently become a full member of the EU

The generic risk assessment matrices, as proposed in the Common Methodology, show potential interactions and conflicts between different fishing gears and the habitats and species for which the site has been designated. A site specific risk assessment matrix takes this further by considering additional factors, such as fishing effort and the spatial distribution of fishing activity, to determine the risk to particular habitats and species within the site. The findings are used to introduce measures.

E.g. the access restriction decree relating to the Voordelta Minister of Agriculture, Nature and Food Quality (2008) Restriction of access to natural areas under Section 20 of the Nature Conservation Act 1998 for the ‘Voordelta’ Natura 2000 area

E.g. beam trawling and mechanical cockle picking by Dutch vessels in marine Natura 2000 sites in the Netherlands, hydraulic dredging for cockles in Dundalk Bay, Ireland

E.g. in the Netherlands beam trawling is subject to Art 6.3 appropriate assessment whereas in Ireland ‘bottom trawling’ is not considered to be a plan or project. This is likely to be because of differences in the way they are managed

the option of exempting generally certain activities, in accordance with the rules in force, from the need for an assessment of the implications for the site concerned does not comply with Article 6(3). Case C98/03 Commission v Germany [2006]

ICES (2011) Fisheries Measures in Protected Areas (FIMPAS) within the Exclusive Economic Zone (EEZ) of the Dutch part of the North Sea: areas outside the 12 nautical miles zone: Proposals. 35pp.

Ministry of Economic Affairs, Agriculture and Innovation (2011) Fishing with the limits of Natura 2000. Agreements for the regulation and development of fisheries and nature conservation in the North Sea Coastal Zone and Vlakte van de Raan Natura 2000 sites.

Jak, R.G. et al., (2008) Conservation objectives for Natura 2000 sites (SACs and SPAs) in the Dutch sector of the North Sea. IMARES Report No. C065/09

ICES (2011) Report of the FIMPAS Workshop 3 Management proposals for Dogger Bank, Cleaver Bank and Frisian Front. ICES CM 2011:ACOM:52

Defra (2013) Revised approach to the management of commercial fisheries in European Marine Sites – overarching policy and delivery document.

A Roadmap towards the Management of Fisheries and Aquaculture Activities in Natura 2000 sites in Ireland (2009)

Marine Institute (in press) A Risk Assessment Framework for Fisheries in natura 2000 sites in Ireland with Case study assessments. June 20th 2013 version.

Marine Institute (2011) Appropriate Assessment of fisheries and aquaculture on Dundalk Bay SAC and SPA.

Marine Institute (in press) A Risk Assessment Framework for Fisheries in Natura 2000 sites in Ireland with Case study assessments. June 20th 2013 version.

The 15% figure is specific to this location and this habitat type. NPWS. 2011. Donegal Bay (Murvagh) SAC (00133): Conservation Objectives supporting document – marine habitats and species. Department Arts, Heritage and the Gaeltacht. Version 1 (November 2011); 17pp.

http://www.fisheriesireland.ie/Fisheries-Management/oyster-dredge-licences.html

E.g. Barcelo, A. et al (2010) Concertation et mode de gouvernance lors de la mise en place de l’aire marine protégée de Porquerolles (Hyéres, France) dans le cadre de Natura 2000 en mer. Sci.Rep. Port-Cros natl.Park, Fr., 24:21-38. http://www.portcrosparcnational.fr/upload/rscientifique/T24_Concertation%20et%20mode%20de%20gouvernance.pdf

MNHN-SPN (2012)  Méthode d’évaluation des risques de dégradation des habitats naturels et des espèces d'intérêt communautaire par les activités de pêche maritime - Résumé -Rapport MNHN-SPN / MEDDE-DPMA. 22 pages

Gesetz zur Regelung der Seefischerei und zur Durchführung des Fischereirechts der Europäischen Union (Seefischereigesetz - SeeFischG), 1998

Seefischereiverordnung (SeefiV), 1989

BNatschG 2009, § 5 (4):  Bei der fischereiwirtschaftlichen Nutzung der oberirdischen Gewässer sind diese einschließlich ihrer Uferzonen als Lebensstätten und Lebensräume für heimische Tier- und Pflanzenarten zu erhalten und zu fördern.

BNatschG, § 14.2: „Entspricht die land-, forst- und fischereiwirtschaftliche Bodennutzung den in § 5 Absatz 2 bis 4 dieses Gesetzes genannten Anforderungen sowie den sich aus § 17 Absatz 2 des Bundes-Bodenschutzgesetzes und dem Recht der Land-, Forst- und Fischereiwirtschaft ergebenden Anforderungen an die gute fachliche Praxis, widerspricht sie in der Regel nicht den Zielen des Naturschutzes und der Landschaftspflege“.

Pedersen et al., (2008); Natura 2000 sites and fisheries in German offshore waters. ICES J.Mar Sci 66:155-169; ICES (2008) The management of Natura 2000 sites in the German EEZ: summary and advice derived from the results of theEMPAS project.

Agrarministerkonferenz am 28. Oktober 2011 in Suhl. Ergebnisprotokoll TOP 33: NATURA 2000 Management in der

deutschen ausschließlichen Wirtschaftszone (AWZ)

Landesverordnung über die Ausübung der Fischerei in den Küstengewässern (Küstenfischereiverordnug - KüFo), 2008

Wadden Sea Board (2013) Framework for Sustainable Fisheries. Annex 5. MCD 2014 – Sustainable fisheries.

http://www.landesrecht-mv.de/jportal/portal/page/bsmvprod.psml?showdoccase=1&doc.id=jlr KüFischVMV2006rahmen&st=lr

Ministerium für Landwirtschaft Umwelt und Verbraucherschutz Mecklenburg-Vorpommern, 2012. Fachleitfaden

„Managementplanung für Natura 2000-Gebiete in Mecklenburg-Vorpommern“. Teil II des Handbuches zur Umsetzung der Fördermaßnahme 323a. Version 3 April 2012 Ministerium für Landwirtschaft, Umwelt und Verbraucherschutz Mecklenburg-Vorpommern, Referat Managementplanung Natura 2000 Gebiete, Landschaftsplanung.

Berg, Möller, 2006. Managementplan für das FFH-Gebiet DE 1934-302 Wismarbucht (gleichzeitig teilweise

Vogelschutzgebiet DE 2034-401 gemäß Vogelschutzrichtlinie. Umweltministerium Mecklenburg-Vorpommern, Referat

Landschaftsplanung, Management der Natura 2000 Gebiete, p. 132.

Lambrecht, H., Trautner, J., 2007. Fachinformationssystem und Fachkonventionen zur Bestimmung der Erheblichkeit imRahmen der FFH-VP. – Endbericht zum Teil Fachkonventionen, Schlussstand Juni 2007. F+E-Vorhaben im Rahmen des Umweltforschungsplanes des Bundesministeriums für Umwelt, Naturschutz und Reaktorsicherheit im Auftrag des

Bundesamtes für Naturschutz - FKZ 804 82 004. Hannover, Filderstadt, p. 239.

Identification and setting-up of the Marine part of Natura 2000 network in Croatia – Marine NATURA 2000. http://www.dzzp.hr/projekti-iz-eu-fondova/ipa/ipa-2007-„identifikacija-i-uspostavljanje-morskog-dijela-mreze-natura-2000-u-hrvatskoj“-803.html

Javna ustanova Park prirode „Lastovsko otočje“ (2012): Plan upravljanja Parkom prirode „Lastovsko otočje“ http://www.pp-lastovo.hr/images/ckfinder/files/Prijedlog%20Plana%20upravljanja%20PP%20Lastovsko%20otocje.pdf

Institut za oceanografiju i ribarstvo (2012): Početna procjena stanja i opterećenja morskog okoliša hrvatskog dijela Jadrana. Document prepared for the Ministry of Environmental and Nature Protection.

http://www.mzoip.hr/doc/More/Pocetna_procjena_morski_okolis.pdf

Royal Decree 1629/2011, of 14 November, on the declaration as a Marine Protected Area and Special Area of Conservation the marine area of El Cachucho, and approval of the appropriate conservation measures. Available at: http://www.magrama.gob.es/es/costas/temas/biodiversidad-marina/RD_1629_2011_El_Cachucho_tcm7-229981.pdf

MINISTERIO DE MEDIO AMBIENTE (2007). Informe de síntesis de los valores ambientales de El Cachucho, Madrid, 41 pp.

Act 42/2007, of 13 December, on Natural Heritage and Biodiversity.